|  | TITLE |
| --- | --- |

| SUBJECT: | MT61-HD0 BIOS Release Notes version F08 |
| --- | --- |
| System: | MT61-HD0-00, MT61-HD0-GB, H261-T60-00, H261-T61-00, TN11-T60-GB, TN11-T60-00, |

# About This Release

Build Date:		2024/08/13

BIOS Checksum:	0x92B21648 (16M), 0x92B21648 (32M)

Release Owner:	Yuan.Chen

# BIOS Components/Contents

Processor stepping(s) supported: Cavium ThunderX2 CN99XX series processors

System hardware configurations supported:  MT61-HD0 Rev: 0.1, 0.2

IPMI support:

AMI Kernel version:

OPROM version :

| OPROM | Version |
| --- | --- |
| AST2500 VGA | 800 |

# Installation Notes

IMPORTANT NOTES:

- Please extract the MT61-HD0_F08.zip

BIOS UPDATE INSTRUCTIONS FOR Web UI

- Connect BMC_IP by Internet browser

- Click Web-UI sub-page button "Update"

- Select Firmware Type to ROM

- Use Command Shell

- cd to /RBU

- Select File Path to image.RBU

- Click Upload

For more details , please reference :

http://download.gigabyte.asia/FileList/BIOS/server_system_flash_bios_guide_1.0.zip

Gigabyte_Cavium_projects_BIOS_update_by_Web_UI_SOP_2016_0413.pdf

BIOS UPDATE INSTRUCTIONS FOR GbtUtility:

1. Please copy the MT61-HD0_F08.zip to GbtUtility folder

2. Use Command Shell

3. cd to GbtUtility folder

4. Execute "java -jar GbtUtility.jar -H BMC_IP update bios MT61-HD0_F08.zip 1"

BIOS UPDATE INSTRUCTIONS FOR Shell AfuEFIAarch64.EFI:

1. Please copy the MT61-HD0_F08 folder to USB

2. Power on the boot into Shell

3. Please insert the USB into the USB port

4. cd MT61-HD0_F08 folder

5. cd /BIOS_Update_Tools/UEFI

6. Execute "./update_bios.nsh image.bin"

7. reset system

BIOS UPDATE INSTRUCTIONS FOR Ubuntu AfulnxAarch64.lnx:

1. Please copy the MT61-HD0_F08 folder to USB

2. Power on the boot into Ubuntu

3. Please insert the USB into the USB port

4. mount usb

4. cd MT61-HD0_F08 folder

5. cd BIOS_Update_Tools/OS

6. Execute ". /update_bios.sh image.bin"

7. reboot system

BIOS Version CHECK INSTRUCTIONS:
1. Power on system and press <ESC> / <F2> during POST
2. The bios version shows on the first main page

# Known Issues/Workarounds

# Issues fixed in this version

1. [Bug Fix] Fix the Security PKfail issue.