|  | TITLE |
| --- | --- |

| SUBJECT: | MT91-FS1 BIOS Release Notes version F37 |
| --- | --- |
| System: | MT91-FS1-00, R181-T90-00, R281-T91-00, W281-T91-GB, W281-T91-00, MT91-FS1-GB, MT91-FS4-00, MT91-FS4-GB, R181-T92-00, R181-T92-GB, R281-T94-00, R281-T94-GB, R281-005-V6, R281-006-V6, R281-T95-00, R281-005-ES-YOUHUA-001, R281-005-ES-YOUHUA-002 |

# About This Release

Build Date:		2024/08/13

BIOS Checksum:	0xA3D80AA4 (16M), 0xA3D80AA4 (32M)

Release Owner:	Yuan.Chen

# BIOS Components/Contents

Processor stepping(s) supported: Cavium ThunderX2 CN99XX series processors

System hardware configurations supported:  MT91-FS1 Rev: 0.1, 0.2, 1.*

IPMI support:

AMI Kernel version:

OPROM version :

| OPROM | Version |
| --- | --- |
| AST2500 VGA | 800 |

# Installation Notes

IMPORTANT NOTES:

- Please extract the MT91-FS1_F37.zip

BIOS UPDATE INSTRUCTIONS FOR Web UI

- Connect BMC_IP by Internet browser

- Click Web-UI sub-page button "Update"

- Select Firmware Type to ROM

- Use Command Shell

- cd to /RBU

- Select File Path to image.RBU

- Click Upload

For more details , please reference :

http://download.gigabyte.asia/FileList/BIOS/server_system_flash_bios_guide_1.0.zip

Gigabyte_Cavium_projects_BIOS_update_by_Web_UI_SOP_2016_0413.pdf

BIOS UPDATE INSTRUCTIONS FOR GbtUtility:

1. Please copy the MT91-FS1_F37.zip to GbtUtility folder

2. Use Command Shell

3. cd to GbtUtility folder

4. Execute "java -jar GbtUtility.jar -H BMC_IP update bios MT91-FS1_F37.zip 1"

BIOS UPDATE INSTRUCTIONS FOR Shell AfuEFIAarch64.EFI:

1. Please copy the MT91-FS1_F37 folder to USB

2. Power on the boot into Shell

3. Please insert the USB into the USB port

4. cd MT91-FS1_F37 folder

5. cd SPI_UPD\UEFI\

6. Execute "update_bios.nsh"

7. reset system

BIOS UPDATE INSTRUCTIONS FOR Ubuntu AfulnxAarch64.lnx:

1. Please copy the MT91-FS1_F37 folder to USB

2. Power on the boot into Ubuntu

3. Please insert the USB into the USB port

4. mount usb

4. cd MT91-FS1_F37 folder

5. cd SPI_UPD/OS

6. Execute "./update_bios.sh"

7. reboot system

BIOS Version CHECK INSTRUCTIONS:
1. Power on system and press <ESC> / <F2> during POST
2. The bios version shows on the first main page

# Known Issues/Workarounds

# Issues fixed in this version

- 1. [Bug Fix] Fix the Security PKfail issue.